# Chipping detection in ceramic insert during turning by analysis of workpiece surface profile using cross-correlation

N A Mohamad1, M M Ratnam1*, Z A Ahmad2

1School of Mechanical Engineering, Engineering Campus, Universiti Sains Malaysia, 14300 Nibong Tebal, Penang, Malaysia.

2School of Materials and Mineral Resources Engineering, Engineering Campus, Universiti Sains Malaysia, 14300 Nibong Tebal, Penang, Malaysia.

*E-mail: mmaran@usm.my

Abstract. Chipping in ceramic cutting inserts during machining can cause detrimental effects on the surface finish quality of the workpiece. Current methods of detecting chipping in ceramic inserts do not compare the minute changes in the workpiece profile in subsequent machining passes. In this work, an effective method of detecting chipping in ceramic inserts by analysing the workpiece surface profile using cross-correlation is proposed. In this method, the sub-pixel edge location method was used to extract the surface profile captured using a digital camera. Surface profiles corresponding to subsequent machining passes were cross-correlated at three different sections of the workpiece. An abrupt drop in the correlation coefficient was observed in the middle and end sections of the workpiece after the fifth machining pass, thus providing a reliable measure of tool chipping during turning.

Keywords: chipping detection, ceramic inserts, turning

Conference sub-theme: Tool condition monitoring